Montage LZ Linux Brief Sample
DVB-C User Guide

Document Number: LZ000446

Revision Number: 0.0

Revision Date: Jan. 8, 2024

Montage LZ Technologies Proprietary and Confidential

Revision History

| Revision Number | Revision Date | Changes |  |
| --- | --- | --- | --- |
|  |  | Page Number | Description |
| 0.0 | Jan. 8, 2024 | - | Initial Release |

Table of Contents

1	Overview	1

2	Sample Demo Information	2

2.1	Flowcharts	2

2.2	Function Modules	3

2.2.1	Switching Programs	3

2.3	Using API	3

2.3.1	mt_sys_init()	3

2.3.2	MTADP_HDMI_Init()	3

2.3.3	MTADP_Disp_Init()	3

2.3.4	MTADP_Fe_Init()	4

2.3.5	MTADP_Fe_Connect_Dvbc()	4

2.3.6	MTADP_VO_Init()	4

2.3.7	MTADP_Snd_Init()	4

2.3.8	MT_UNF_DMX_Init()	4

2.3.9	MT_UNF_DMX_AttachTSPort()	4

2.3.10	MTADP_Search_GetAllPmt()	4

2.3.11	MT_UNF_AVPLAY_Init ()	4

2.3.12	MT_UNF_AVPLAY_Create()	5

2.3.13	MT_UNF_AVPLAY_ChnOpen()	5

2.3.14	MT_UNF_SND_CreateTrack()	5

2.3.15	MT_UNF_SND_Attach()	5

2.3.16	MTADP_VO_CreatWin()	5

2.3.17	MT_UNF_VO_AttachWindow()	5

2.3.18	MT_UNF_AVPLAY_Start()	6

2.3.19	MT_UNF_AVPLAY_Stop()	6

2.3.20	MT_UNF_VO_DetachWindow()	6

2.3.21	MT_UNF_VO_DestroyWindow()	6

2.3.22	MT_UNF_SND_Detach()	6

2.3.23	MT_UNF_SND_DestroyTrack()	6

2.3.24	MT_UNF_AVPLAY_ChnClose()	6

2.3.25	MT_UNF_AVPLAY_Destroy()	7

2.3.26	MT_UNF_AVPLAY_DeInit()	7

2.3.27	MTADP_Search_FreeAllPmt()	7

2.3.28	MT_UNF_DMX_DetachTSPort()	7

2.3.29	MT_UNF_DMX_DeInit()	7

2.3.30	MTADP_Snd_DeInit()	7

2.3.31	MTADP_VO_DeInit()	7

2.3.32	MTADP_Fe_DeInit()	8

2.3.33	MTADP_Disp_DeInit()	8

2.3.34	MTADP_HDMI_DeInit()	8

2.3.35	mt_sys_deinit()	8

3	Compilation	9

4	How to Use Sample Demo	10

4.1	Test Sample Environment Preparation	10

4.2	Test Single Board Environment Preparation	10

5	Supported Sample Commands	11

# 1	Overview

This sample primarily implements the functionality of DVB-C tuner frequency locking, channel search, playback, switching channels.

# 2	Sample demo Information

## 2.1	Flowcharts

## 2.2	Function Modules

### 2.2.1   Switching Programs

Input program number to switching programs.

### 2.2.2   Signal Strength

Input command ‘s’ to print signal strength info.

### 2.2.3   Signal Quality

Input command ‘l’ to print signal quality info.

### 2.2.4   Pause

Input command ‘p’ to pause the current playback.

### 2.2.5   Resume

Input command ‘r’ to resume the normal playback.

## 2.3	Using API

### 2.3.1	mt_sys_init()

[Information]

System device initialization; must be called, or various devices may not work correctly.

### 2.3.2	MTADP_HDMI_Init()

[Information]

Initialize HDMI.

### 2.3.3	MTADP_Disp_Init()

[Information]

Initialize display.

### 2.3.4	MTADP_Fe_Init()

[Information]

Initialize tuner device.

### 2.3.5	MTADP_Fe_Connect_Dvbc()

[Information]

Connect DVB-C signal.

### 2.3.6	MTADP_VO_Init()

[Information]

Initialize VO device.

### 2.3.7	MTADP_Snd_Init()

[Information]

Initialize sound device.

### 2.3.8	MT_UNF_DMX_Init()

[Information]

Initialize Demux.

### 2.3.9	MT_UNF_DMX_AttachTSPort()

[Information]

Attach Demux to tuner device.

### 2.3.10	MTADP_Search_GetAllPmt()

[Information]

Retrieve PMT and PAT tables in TS.

### 2.3.11	MT_UNF_AVPLAY_Init ()

[Information]

Initialize AV player.

### 2.3.12	MT_UNF_AVPLAY_Create()

[Information]

Create AV player.

### 2.3.13	MT_UNF_AVPLAY_ChnOpen()

[Information]

Open audio and video channels.

### 2.3.14	MT_UNF_SND_CreateTrack()

[Information]

Create audio track.

### 2.3.15	MT_UNF_SND_Attach()

[Information]

Attach audio channel.

### 2.3.16	MTADP_VO_CreatWin()

[Information]

Create a window.

### 2.3.17	MT_UNF_VO_AttachWindow()

[Information]

Attach window.

### 2.3.18	MT_UNF_AVPLAY_Start()

[Information]

Start playing video.

### 2.3.19	MT_UNF_AVPLAY_Stop()

[Information]

Stop playing video.

### 2.3.20	MT_UNF_VO_DetachWindow()

[Information]

Detach window from player.

### 2.3.21	MT_UNF_VO_DestroyWindow()

[Information]

Destroy window.

### 2.3.22	MT_UNF_SND_Detach()

[Information]

Detach audio channel connection.

### 2.3.23	MT_UNF_SND_DestroyTrack()

[Information]

Destroy audio track.

### 2.3.24	MT_UNF_AVPLAY_ChnClose()

[Information]

Close audio and video channels.

### 2.3.25	MT_UNF_AVPLAY_Destroy()

[Information]

Destroy AV player.

### 2.3.26	MT_UNF_AVPLAY_DeInit()

[Information]

Deinitialize AV player.

### 2.3.27	MTADP_Search_FreeAllPmt()

[Information]

Release PMT and PAT tables in TS.

### 2.3.28	MT_UNF_DMX_DetachTSPort()

[Information]

Detach Demux and tuner.

### 2.3.29	MT_UNF_DMX_DeInit()

[Information]

Deinitialize Demux.

### 2.3.30	MTADP_Snd_DeInit()

[Information]

Deinitialize audio device.

### 2.3.31	MTADP_VO_DeInit()

[Information]

Deinitialize VO device.

### 2.3.32	MTADP_Fe_DeInit()

[Information]

Deinitialize Tuner.

### 2.3.33	MTADP_Disp_DeInit()

[Information]

Deinitialize display.

### 2.3.34	MTADP_HDMI_DeInit()

[Information]

Deinitialize HDMI.

### 2.3.35	mt_sys_deinit()

[Information]

Deinitialize system.

# 3	Compilation

Execute ‘make’ for compilation in the dvbc directory.

# 4	How to use sample demo

## 4.1	Test Sample Environment Preparation

Connect the antenna.

Copy sample_dvbc to a USB drive: linux\out\general\breif_sample\dvbc\ sample_dvbc.

Copy library files to the USB drive: copy the directory linux\pub\shared_lib_striped to the root directory of the USB drive.

4.2	Test Single Board Environment Preparation

Connect the USB drive to the single board.

Execute the following commands:

mount /dev/sda1 /mnt/

export LD_LIBRARY_PATH=/mnt/shared_lib_striped/:/usr/local/lib

cd /mnt/

./sample_dvbc -f 314 -s 6875 -p 64 //-f Frequency -s Symbol rate -p QAM

# 5	Supported Sample Commands

| Command | Description |
| --- | --- |
| q | Quit |
| h | Print Help info |
| s | Print signal strength info |
| l | Print signal quality info |
| p | Pause |
| r | Resume |