Montage LZ Linux Brief Sample
DVB-S User Guide

Document Number: LZ000447

Revision Number: 0.0

Revision Date: Jan. 8, 2024

Montage LZ Technologies Proprietary and Confidential

Revision History

| Revision Number | Revision Date | Changes |  |
| --- | --- | --- | --- |
|  |  | Page Number | Description |
| 0.0 | Jan. 8, 2024 | - | Initial Release |

Table of Contents

1	Overview	1

2	Sample Demo Information	2

2.1	Flowcharts	2

2.2	Function Modules	3

2.2.1	Signal Strength	3

2.2.2	Signal Quality	3

2.2.3	Pause	3

2.2.4	Resume	3

2.2.5	Set Whether to Reserve the Last Frame	3

2.2.6	Switching Programs	3

2.3	Using API	3

2.3.1	mt_sys_init()	3

2.3.2	MTADP_HDMI_Init ()	4

2.3.3	MTADP_Disp_Init ()	4

2.3.4	MTADP_Snd_Init ()	4

2.3.5	MTADP_VO_Init ()	4

2.3.7	MTADP_Fe_Init ()	4

2.3.7	MTADP_Fe_Connect_Dvbs ()	4

2.3.8	MT_UNF_DMX_Init ()	4

2.3.9	MT_UNF_DMX_AttachTSPort ()	5

2.3.10	MTADP_Search_Init ()	5

2.3.11	MTADP_Search_GetAllPmt ()	5

2.3.12	MT_UNF_AVPLAY_Init ()	5

2.3.13	MT_UNF_AVPLAY_Create()	5

2.3.14	MT_UNF_SND_CreateTrack()	5

2.3.15	MTADP_VO_CreatWin()	5

2.3.16	MT_UNF_VO_SetWindowEnable ()	6

2.3.17	MT_UNF_AVPLAY_Start ()	6

2.3.18	MT_UNF_AVPLAY_Stop ()	6

2.3.19	MT_UNF_VO_SetWindowEnable ()	6

2.3.20	MT_UNF_VO_DetachWindow ()	6

2.3.21	MT_UNF_VO_DestroyWindow ()	6

2.3.22	MT_UNF_SND_DestroyTrack ()	6

2.3.23	MT_UNF_AVPLAY_ChnClose ()	7

2.3.24	MT_UNF_AVPLAY_Destroy ()	7

2.3.25	MT_UNF_AVPLAY_DeInit ()	7

2.3.26	MTADP_Search_FreeAllPmt ()	7

2.3.27	MTADP_Search_DeInit ()	7

2.3.28	MT_UNF_DMX_DetachTSPort ()	7

2.3.29	MT_UNF_DMX_DeInit ()	7

2.3.30	MTADP_Snd_DeInit ()	8

2.3.31	MTADP_VO_DeInit ()	8

2.3.32	MTADP_Disp_DeInit ()	8

2.3.33	MTADP_HDMI_DeInit ()	8

2.3.34	mt_sys_deinit ()	8

3	Compilation	9

4	How to Use Sample Demo	10

4.1	Test Sample Environment Preparation	10

4.2	Test Single Board Environment Preparation	10

5	Supported Sample Commands	11

# 1	Overview

This sample primarily implements the functionality of DVB-S frequency locking and playing programs, providing guidance for customers when using DVB-S functions, and providing simplified and procedural call references in Debug.

# 2	Sample Demo Information

## 2.1	Flowcharts

## 2.2	Function Modules

### 2.2.1	Signal Strength

Input command ‘s’ to get signal strength info.

Invoke function mt_unf_fe_get_signal_strength ().

### 2.2.2	Signal Quality

Input command ‘l’ to get signal quality info.

Invoke function mt_unf_fe_get_signal_quality ().

### 2.2.3	Pause

Input command ‘p’ to pause the current payback.

### 2.2.4	Resume

Input command ‘r’ to resume the normal playback.

### 2.2.5	Set Whether to Reserve the Last Frame

Input command ‘z’ to set the mode after playback is stopped.

### 2.2.6	Switching Programs

Input program number to play different programs.

Invoke function MT_UNF_AVPLAY_Start () to play the program.

## 2.3	Using API

### 2.3.1	mt_sys_init()

[Information]

System device initialization; must be called, or various devices may not work correctly.

### 2.3.2	MTADP_HDMI_Init ()

[Information]

Initialize HDMI.

### 2.3.3	MTADP_Disp_Init ()

[Information]

Initialize display.

### 2.3.4	MTADP_Snd_Init ()

[Information]

Initialize sound device.

### 2.3.5	MTADP_VO_Init ()

[Information]

Initialize VO device.

### 2.3.7	MTADP_Fe_Init ()

[Information]

Initialize tuner device.

### 2.3.7	MTADP_Fe_Connect_Dvbs ()

[Information]

Connect DVB-S.

### 2.3.8	MT_UNF_DMX_Init ()

[Information]

Initialize Demux.

### 2.3.9	MT_UNF_DMX_AttachTSPort ()

[Information]

Attach Demux to tuner device.

### 2.3.10	MTADP_Search_Init ()

[Information]

Initialize search.

### 2.3.11	MTADP_Search_GetAllPmt ()

[Information]

Retrieve PMT and PAT tables in TS.

### 2.3.12	MT_UNF_AVPLAY_Init ()

[Information]

Initialize AV player.

### 2.3.13	MT_UNF_AVPLAY_Create()

[Information]

Create AV player.

### 2.3.14	MT_UNF_SND_CreateTrack()

[Information]

Create audio track.

### 2.3.15	MTADP_VO_CreatWin()

[Information]

Create a window.

### 2.3.16	MT_UNF_VO_SetWindowEnable ()

[Information]

Open window display.

### 2.3.17	MT_UNF_AVPLAY_Start ()

[Information]

Start playing video.

### 2.3.18	MT_UNF_AVPLAY_Stop ()

[Information]

Stop playing video.

### 2.3.19	MT_UNF_VO_SetWindowEnable ()

[Information]

Close window display.

### 2.3.20	MT_UNF_VO_DetachWindow ()

[Information]

Detach window from player.

### 2.3.21	MT_UNF_VO_DestroyWindow ()

[Information]

Destroy window.

### 2.3.22	MT_UNF_SND_DestroyTrack ()

[Information]

Destroy audio track.

### 2.3.23	MT_UNF_AVPLAY_ChnClose ()

[Information]

Close audio and video channels.

### 2.3.24	MT_UNF_AVPLAY_Destroy ()

[Information]

Destroy AV player.

### 2.3.25	MT_UNF_AVPLAY_DeInit ()

[Information]

Deinitialize AV player.

### 2.3.26	MTADP_Search_FreeAllPmt ()

[Information]

Release PMT and PAT tables in TS.

### 2.3.27	MTADP_Search_DeInit ()

[Information]

Deinitialize search.

### 2.3.28	MT_UNF_DMX_DetachTSPort ()

[Information]

Detach Demux and tuner.

### 2.3.29	MT_UNF_DMX_DeInit ()

[Information]

Deinitialize Demux.

### 2.3.30	MTADP_Snd_DeInit ()

[Information]

Deinitialize audio device.

### 2.3.31	MTADP_VO_DeInit ()

[Information]

Deinitialize VO device.

### 2.3.32	MTADP_Disp_DeInit ()

[Information]

Deinitialize display.

### 2.3.33	MTADP_HDMI_DeInit ()

[Information]

Deinitialize HDMI.

### 2.3.34	mt_sys_deinit ()

[Information]

Deinitialize system.

# 3	Compilation

Execute ‘make’ for compilation.

# 4	How to Use Sample Demo

## 4.1	Test Sample Environment Preparation

Copy sample_dvbs to a USB drive: linux\out\general\breif_sample\dvbs.

Copy library files to the USB drive: copy the directory linux\pub\shared_lib_striped to the root directory of the U drive.

## 4.2	Test Single Board Environment Preparation

Connect the USB drive to the single board.

Execute the following commands:

mount /dev/sda1 /mnt/

export LD_LIBRARY_PATH=/mnt/shared_lib_striped/:/usr/local/lib

cd /mnt

./sample_dvbs–f 3840 –s 27500–k 1–p 0 –t 0

The 5 parameters are tunerFreq, tunerSrate, onoff_22k, polar, dvb_type.

# 5	Supported Sample Commands

| Command | Description |
| --- | --- |
| q | Quit sample dvbs demo |
| h | Print support command |
| p | Pause playback |
| r | Playback resumes |
| z | Set the mode after playback is stopped |
| s | signal strength |
| l | signal quality |
| Programme Number | Switch programs |