Montage LZ Linux Brief Sample
PVR Play User Guide

Document Number: LZ000449

Revision Number: 0.0

Revision Date: Jan. 8, 2024

Montage LZ Technologies Proprietary and Confidential

Revision History

| Revision Number | Revision Date | Changes |  |
| --- | --- | --- | --- |
|  |  | Page Number | Description |
| 0.0 | Jan. 8, 2024 | - | Initial Release |

Table of Contents

1	Overview	1

2	Sample Demo Information	2

2.1	Flowcharts	2

2.2	Function Modules	3

2.2.1	Fast Playback	3

2.2.2	Fast Backward	3

2.2.3	Slow Forward	3

2.2.4	Slow Backward	3

2.2.5	Normal Playback	3

2.2.6	Pause	3

2.2.7	Seek to Start	3

2.2.8	Seek to End	3

2.2.9	Seek Forward	3

2.2.10	Seek Rewind	4

2.3	Using API	4

2.3.1	mt_sys_init()	4

2.3.2	MTADP_HDMI_Init ()	4

2.3.3	MTADP_Disp_Init()	4

2.3.4	MTADP_VO_Init()	4

2.3.5	MTADP_Snd_Init()	4

2.3.6	MT_UNF_DMX_Init()	4

2.3.7	MT_UNF_DMX_AttachTSPort()	5

2.3.8	MT_UNF_AVPLAY_Init()	5

2.3.9	MT_UNF_AVPLAY_Create()	5

2.3.10	MT_UNF_SND_CreateTrack()	5

2.3.11	MTADP_VO_CreatWin()	5

2.3.12	MT_UNF_VO_SetWindowEnable ()	5

2.3.13	MT_UNF_PVR_PlayInit()	5

2.3.14	MTADP_PVR_RegisterCallBacks()	6

2.3.15	MTADP_PVR_StartPlayBack()	6

2.3.16	MT_UNF_PVR_PlayGetStatus()	6

2.3.17	MT_UNF_PVR_PlayGetFileAttr()	6

2.3.18	MT_UNF_PVR_PlayTPlay()	6

2.3.19	MT_UNF_PVR_PlayResumeChn()	6

2.3.20	MT_UNF_PVR_PlayPauseChn()	6

2.3.21	MT_UNF_PVR_PlaySeek ()	7

2.3.22	MTADP_PVR_StopPlayBack()	7

2.3.23	MT_UNF_PVR_PlayDeInit()	7

2.3.24	MT_UNF_VO_SetWindowEnable ()	7

2.3.25	MT_UNF_VO_DetachWindow ()	7

2.3.26	MT_UNF_VO_DestroyWindow ()	7

2.3.27	MT_UNF_SND_DestroyTrack ()	7

2.3.28	MT_UNF_AVPLAY_ChnClose ()	8

2.3.29	MT_UNF_AVPLAY_Destroy ()	8

2.3.30	MT_UNF_AVPLAY_DeInit ()	8

2.3.31	MT_UNF_DMX_DetachTSPort ()	8

2.3.32	MT_UNF_DMX_DeInit ()	8

2.3.33	MTADP_Snd_DeInit ()	8

2.3.34	MTADP_VO_DeInit ()	8

2.3.35	MTADP_Disp_DeInit ()	9

2.3.36	MTADP_HDMI_DeInit ()	9

2.3.37	mt_sys_deinit ()	9

3	Compilation	10

4	How to Use Sample Demo	11

4.1	Test Sample Environment Preparation	11

4.2	Test Single Board Environment Preparation	11

5	Supported Sample Commands	12

# 1	Overview

This sample primarily implements the functionality of playing the recorded live programs and some special playback.

# 2	Sample Demo Information

## 2.1	Flowcharts

## 2.2	Function Modules

### 2.2.1	Fast Playback

Input command ‘f’ continuously to increase the playback speed up to 32 times.

### 2.2.2	Fast Backward

Input command ‘r’ continuously to increase the backward speed up to 32 times.

### 2.2.3	Slow Forward

Input command ‘s’ to decrease the playback speed down to 1/4 times.

### 2.2.4	Slow Backward

Input command ‘g’ to decrease the backward speed down to 1/32 times.

### 2.2.5	Normal Playback

Input command ‘n’ to resume the normal playback.

### 2.2.6	Pause

Input command ‘p’ to pause the playback.

### 2.2.7	Seek to Start

Input command ‘k’ to seek to start.

### 2.2.8	Seek to End

Input command ‘e’ to seek to end.

### 2.2.9	Seek Forward

Input command ‘d’ to seek forward 5 s.

### 2.2.10	Seek Rewind

Input command ‘a’ to seek rewind 5 s.

## 2.3	Using API

### 2.3.1	mt_sys_init()

[Information]

System device initialization; must be called, or various devices may not work correctly.

### 2.3.2	MTADP_HDMI_Init ()

[Information]

Initialize HDMI.

### 2.3.3	MTADP_Disp_Init()

[Information]

Initialize display.

### 2.3.4	MTADP_VO_Init()

[Information]

Initialize VO device.

### 2.3.5	MTADP_Snd_Init()

[Information]

Initialize sound device.

### 2.3.6	MT_UNF_DMX_Init()

[Information]

Initialize Demux.

### 2.3.7	MT_UNF_DMX_AttachTSPort()

[Information]

Attach Demux to tuner device.

### 2.3.8	MT_UNF_AVPLAY_Init()

[Information]

Initialize AV player.

### 2.3.9	MT_UNF_AVPLAY_Create()

[Information]

Create AV player.

### 2.3.10	MT_UNF_SND_CreateTrack()

[Information]

Create audio track.

### 2.3.11	MTADP_VO_CreatWin()

[Information]

Create a window.

### 2.3.12	MT_UNF_VO_SetWindowEnable ()

[Information]

Open window display.

### 2.3.13	MT_UNF_PVR_PlayInit()

[Information]

Initialize PVR play module.

### 2.3.14	MTADP_PVR_RegisterCallBacks()

[Information]

Register callback functions.

### 2.3.15	MTADP_PVR_StartPlayBack()

[Information]

Start playing back.

### 2.3.16	MT_UNF_PVR_PlayGetStatus()

[Information]

Get playing status.

### 2.3.17	MT_UNF_PVR_PlayGetFileAttr()

[Information]

Get playing file attribute.

### 2.3.18	MT_UNF_PVR_PlayTPlay()

[Information]

Playing speed.

### 2.3.19	MT_UNF_PVR_PlayResumeChn()

[Information]

Resume playing channel.

### 2.3.20	MT_UNF_PVR_PlayPauseChn()

[Information]

Pause playing channel.

### 2.3.21	MT_UNF_PVR_PlaySeek ()

[Information]

Seek to the given location and play.

### 2.3.22	MTADP_PVR_StopPlayBack()

[Information]

Stop playback.

### 2.3.23	MT_UNF_PVR_PlayDeInit()

[Information]

Deinitialize PVR playing module.

### 2.3.24	MT_UNF_VO_SetWindowEnable ()

[Information]

Close window display.

### 2.3.25	MT_UNF_VO_DetachWindow ()

[Information]

Detach window from player.

### 2.3.26	MT_UNF_VO_DestroyWindow ()

[Information]

Destroy window.

### 2.3.27	MT_UNF_SND_DestroyTrack ()

[Information]

Destroy audio track.

### 2.3.28	MT_UNF_AVPLAY_ChnClose ()

[Information]

Close audio and video channels.

### 2.3.29	MT_UNF_AVPLAY_Destroy ()

[Information]

Destroy AV player.

### 2.3.30	MT_UNF_AVPLAY_DeInit ()

[Information]

Deinitialize AV player.

### 2.3.31	MT_UNF_DMX_DetachTSPort ()

[Information]

Detach Demux and tuner.

### 2.3.32	MT_UNF_DMX_DeInit ()

[Information]

Deinitialize Demux.

### 2.3.33	MTADP_Snd_DeInit ()

[Information]

Deinitialize audio device.

### 2.3.34	MTADP_VO_DeInit ()

[Information]

Deinitialize VO device.

### 2.3.35	MTADP_Disp_DeInit ()

[Information]

Deinitialize display.

### 2.3.36	MTADP_HDMI_DeInit ()

[Information]

Deinitialize HDMI.

### 2.3.37	mt_sys_deinit ()

[Information]

Deinitialize system.

# 3	Compilation

Execute ‘make  -f sample_pvr_play.mk’ for compilation.

# 4	How to Use Sample Demo

## 4.1	Test Sample Environment Preparation

Copy sample_pvr_play to a USB drive: linux\out\general\breif_sample\pvr\sample_pvr_play.

Copy library files to the USB drive: copy the directory linux\pub\shared_lib_striped to the root directory of the USB drive.

Record the live programs and copy the recorded files (including the recorded index files) to the USB drive.

## 4.2	Test Single Board Environment Preparation

Connect the USB drive to the single board.

Execute the following commands:

mount /dev/sda1 /mnt/

export LD_LIBRARY_PATH=/mnt/shared_lib_striped/:/usr/local/lib

cd /mnt

./sample_pvr_play -f test.ts    //-f Name of the recorded program ( It can only play TS stream, which can be recorded using sample_pvr_rec)

# 5	Supported Sample Commands

| Command | Description |
| --- | --- |
| q | Quit sample timeshift demo |
| h | Print support command |
| f | Fast playback |
| r | Fast backward |
| s | Slow forward |
| g | Slow backward |
| n | Normal play |
| p | Pause and play |
| k | Seek to start |
| e | Seek to end |
| d | Seek forward 5 second |
| a | Seek rewind 5 second |