Montage LZ Linux Brief Sample

PVR Timeshift User Guide

Document Number: LZ000451

Revision Number: 0.0

Revision Date: January. 5, 2024

Montage LZ Technologies Proprietary and Confidential

Revision History

| Revision Number | Revision Date | Changes |  |
| --- | --- | --- | --- |
|  |  | Page Number | Description |
| 0.0 | January. 5, 2024 | - | Initial Release |

Table of Contents

1	Overview	1

2	Sample Demo Information	2

2.1	Flowcharts	2

2.2	Function Modules	3

2.2.1	Fast Forward Mode	3

2.2.2	Rewind Mode	3

2.2.3	Slow Forward Mode	3

2.2.4	Slow Rewind Mode	3

2.2.5	Restore Normal Playback	3

2.2.6	Pause Mode and Start Recording	3

2.2.7	Start Playback from the Beginning	3

2.2.8	Jump to the End	3

2.2.9	Jump Forward Frames	3

2.2.10	Jump Backward Frames	4

2.2.11	Record	4

2.2.12	Stop Recording	4

2.2.13	Set DIO	4

2.2.14	Return to Live	4

2.2.15	Recording Subtitle/Teletext	4

2.2.16	Set Encryption Method	4

2.2.17	Channel Switch	4

2.3	Using API	4

2.3.1	mt_sys_init()	4

2.3.2	MTADP_Fe_Init()	5

2.3.3	MTADP_Fe_Connect_Dvbc()	5

2.3.4	MTADP_Fe_Connect_Dvbs()	5

2.3.5	MTADP_HDMI_Init ()	5

2.3.6	MTADP_Disp_Init()	5

2.3.7	MTADP_VO_Init()	5

2.3.8	MTADP_Snd_Init()	5

2.3.9	MT_UNF_DMX_Init()	6

2.3.10	MT_UNF_DMX_AttachTSPort()	6

2.3.11	MTADP_Search_Init()	6

2.3.12	MTADP_Search_GetAllPmt()	6

2.3.13	MT_UNF_AVPLAY_Init()	6

2.3.14	MT_UNF_AVPLAY_Create()	6

2.3.15	MT_UNF_SND_CreateTrack()	6

2.3.16	MTADP_VO_CreatWin()	7

2.3.17	MT_UNF_VO_SetWindowEnable ()	7

2.3.18	MT_UNF_AVPLAY_Start ()	7

2.3.19	MT_UNF_PVR_RecInit ()	7

2.3.20	MT_UNF_PVR_PlayInit ()	7

2.3.21	MTADP_PVR_RegisterCallBacks ()	7

2.3.22	MTADP_PVR_StopPlayBack ()	7

2.3.23	MT_UNF_PVR_PlayPauseChn ()	8

2.3.24	MT_UNF_PVR_PlayResumeChn ()	8

2.3.25	MT_UNF_PVR_PlaySeek ()	8

2.3.26	MT_UNF_PVR_PlayTPlay ()	8

2.3.27	MTADP_PVR_StartPlayBack ()	8

2.3.28	MT_UNF_PVR_PlayDeInit ()	8

2.3.29	MT_UNF_PVR_RecDeInit ()	8

2.3.30	MTADP_PVR_RecStart ()	9

2.3.31	MTADP_PVR_RecStop ()	9

2.3.32	MT_UNF_AVPLAY_Stop ()	9

2.3.33	MT_UNF_VO_SetWindowEnable ()	9

2.3.34	MT_UNF_VO_DetachWindow ()	9

2.3.35	MT_UNF_VO_DestroyWindow ()	9

2.3.36	MT_UNF_SND_DestroyTrack ()	9

2.3.37	MT_UNF_AVPLAY_ChnClose ()	10

2.3.38	MT_UNF_AVPLAY_Destroy ()	10

2.3.39	MT_UNF_AVPLAY_DeInit ()	10

2.3.40	MTADP_Search_FreeAllPmt ()	10

2.3.41	MTADP_Search_DeInit ()	10

2.3.42	MT_UNF_DMX_DetachTSPort ()	10

2.3.43	MT_UNF_DMX_DeInit ()	10

2.3.44	MTADP_Snd_DeInit ()	11

2.3.45	MTADP_VO_DeInit ()	11

2.3.46	MTADP_Disp_DeInit ()	11

2.3.47	MTADP_HDMI_DeInit ()	11

2.3.48	MTADP_Fe_Deinit ()	11

2.3.49	mt_sys_deinit ()	11

3	Compilation	12

4	How to Use Sample Demo	13

4.1	Test Sample Environment Preparation	13

4.2	Test Single Board Environment Preparation	13

5	Supported Sample Commands	14

# 1	Overview

This sample primarily implements the timeshift functionality of PVR. It provides guidance for clients when using the related functionalities. Additionally, it offers simplified and procedural invocation references in Debug.

# 2	Sample Demo Information

## 2.1	Flowcharts

## 2.2	Function Modules

### 2.2.1	Fast Forward Mode

Enter the command "f" to fast forward. Continuous input increases the playback speed up to 64 times.

### 2.2.2	Rewind Mode

Enter the command "r" to rewind. Continuous input increases the rewind speed up to 64 times.

### 2.2.3	Slow Forward Mode

Enter the command "s" to slow forward.

### 2.2.4	Slow Rewind Mode

Enter the command "g" to slow rewind.

### 2.2.5	Restore Normal Playback

Enter the command "n" to restore normal playback.

### 2.2.6	Pause Mode and Start Recording

Enter the command "p" to start recording and playback.

### 2.2.7	Start Playback from the Beginning

Enter the command "k" to jump to the beginning of playback.

### 2.2.8	Jump to the End

Enter the command "e" to jump to the end of playback.

### 2.2.9	Jump Forward Frames

Enter the command "d" to jump forward by 5 seconds.

### 2.2.10	Jump Backward Frames

Enter the command "a" to jump backward by 5 seconds.

### 2.2.11	Record

Enter the command "w(0,1,2)" to start recording.

### 2.2.12	Stop Recording

Enter the command "t(0,1,2)" to stop recording the program.

### 2.2.13	Set DIO

Enter the command "y" to set DIO.

### 2.2.14	Return to Live

Enter the command "l" to return to live content.

### 2.2.15	Recording Subtitle/Teletext

Enter the command "i" to set recording subtitle, teletext, and other information.

### 2.2.16	Set Encryption Method

Enter the command "m" to set the recording encryption method.

### 2.2.17	Channel Switch

Enter the program's numeric code to perform a channel switch operation.

## 2.3	Using API

### 2.3.1	mt_sys_init()

[Information]

System device initialization; must be called, or various devices may not work correctly.

### 2.3.2	MTADP_Fe_Init()

[Information]

Initialize tuner devices.

### 2.3.3	MTADP_Fe_Connect_Dvbc()

[Information]

About DVB-C frequency locking.

### 2.3.4	MTADP_Fe_Connect_Dvbs()

[Information]

About DVB-S frequency locking.

### 2.3.5	MTADP_HDMI_Init ()

[Information]

HDMI initialization.

### 2.3.6	MTADP_Disp_Init()

[Information]

Display device initialization.

### 2.3.7	MTADP_VO_Init()

[Information]

VO device initialization.

### 2.3.8	MTADP_Snd_Init()

[Information]

Sound device initialization.

### 2.3.9	MT_UNF_DMX_Init()

[Information]

DMX initialization.

### 2.3.10	MT_UNF_DMX_AttachTSPort()

[Information]

Connect DMX to tuner device.

### 2.3.11	MTADP_Search_Init()

[Information]

Initialize search.

### 2.3.12	MTADP_Search_GetAllPmt()

[Information]

Retrieve PMT and PAT tables in TS.

### 2.3.13	MT_UNF_AVPLAY_Init()

[Information]

AV player initialization.

### 2.3.14	MT_UNF_AVPLAY_Create()

[Information]

Create AV player.

### 2.3.15	MT_UNF_SND_CreateTrack()

[Information]

Create audio output channel.

### 2.3.16	MTADP_VO_CreatWin()

[Information]

Create a window.

### 2.3.17	MT_UNF_VO_SetWindowEnable ()

[Information]

Open window display.

### 2.3.18	MT_UNF_AVPLAY_Start ()

[Information]

Start playing video.

### 2.3.19	MT_UNF_PVR_RecInit ()

[Information]

Initialize PVR recording module.

### 2.3.20	MT_UNF_PVR_PlayInit ()

[Information]

Initialize PVR playback module.

### 2.3.21	MTADP_PVR_RegisterCallBacks ()

[Information]

Register callback functions.

### 2.3.22	MTADP_PVR_StopPlayBack ()

[Information]

Stop playback.

### 2.3.23	MT_UNF_PVR_PlayPauseChn ()

[Information]

Pause playback channel.

### 2.3.24	MT_UNF_PVR_PlayResumeChn ()

[Information]

Resume playback channel.

### 2.3.25	MT_UNF_PVR_PlaySeek ()

[Information]

Jump to a specified point to play.

### 2.3.26	MT_UNF_PVR_PlayTPlay ()

[Information]

Play at a specified speed.

### 2.3.27	MTADP_PVR_StartPlayBack ()

[Information]

Start playback.

### 2.3.28	MT_UNF_PVR_PlayDeInit ()

[Information]

Deinitialize PVR playback module.

### 2.3.29	MT_UNF_PVR_RecDeInit ()

[Information]

Deinitialize PVR recording module.

### 2.3.30	MTADP_PVR_RecStart ()

[Information]

Start recording.

### 2.3.31	MTADP_PVR_RecStop ()

[Information]

Stop recording.

### 2.3.32	MT_UNF_AVPLAY_Stop ()

[Information]

Stop playing video.

### 2.3.33	MT_UNF_VO_SetWindowEnable ()

[Information]

Disable window display.

### 2.3.34	MT_UNF_VO_DetachWindow ()

[Information]

Detach window from player.

### 2.3.35	MT_UNF_VO_DestroyWindow ()

[Information]

Destroy window.

### 2.3.36	MT_UNF_SND_DestroyTrack ()

[Information]

Destroy audio track.

### 2.3.37	MT_UNF_AVPLAY_ChnClose ()

[Information]

Close audio and video channels.

### 2.3.38	MT_UNF_AVPLAY_Destroy ()

[Information]

Destroy AV player.

### 2.3.39	MT_UNF_AVPLAY_DeInit ()

[Information]

Deinitialize AV player.

### 2.3.40	MTADP_Search_FreeAllPmt ()

[Information]

Release PMT and PAT tables in TS.

### 2.3.41	MTADP_Search_DeInit ()

[Information]

Deinitialize search.

### 2.3.42	MT_UNF_DMX_DetachTSPort ()

[Information]

Detach DMX and tuner.

### 2.3.43	MT_UNF_DMX_DeInit ()

[Information]

Deinitialize DMX.

### 2.3.44	MTADP_Snd_DeInit ()

[Information]

Deinitialize audio devices.

### 2.3.45	MTADP_VO_DeInit ()

[Information]

Deinitialize VO devices.

### 2.3.46	MTADP_Disp_DeInit ()

[Information]

Deinitialize display.

### 2.3.47	MTADP_HDMI_DeInit ()

[Information]

Deinitialize HDMI.

### 2.3.48	MTADP_Fe_Deinit ()

[Information]

Deinitialize Tuner.

### 2.3.49	mt_sys_deinit ()

[Information]

Deinitialize system.

# 3	Compilation

Execute make -f sample_pvr_timeshift.mk for compilation.

# 4	How to Use Sample Demo

## 4.1	Test Sample Environment Preparation

Copy ‘sample_pvr_timeshift’ to a USB drive: linux\out\general\breif_sample\pvr\ sample_pvr_timeshift

Copy library files to the USB drive, copy the entire linux\pub\shared_lib_striped directory to the root of the USB drive.

## 4.2	Test Single Board Environment Preparation

Connect the USB drive to the single board.

Execute the following commands:

mount /dev/sda1 /mnt/

export LD_LIBRARY_PATH=/mnt/shared_lib_striped/:/usr/local/lib

cd /mnt

./ sample_pvr_timeshift -f ./pvr -c 314 6875 64

./ sample_pvr_timeshift -f ./pvr -s 3840 27500 1 0 0

# 5	Supported Sample Commands

| Command | Description |
| --- | --- |
| q | Quit sample timeshift demo |
| h | Print support command |
| W | Start recording |
| l | Back to live streaming |
| f | Fast playback |
| r | Fast backward |
| s | Slow forward |
| g | Slow backward |
| m | Set whether to encrypt recording |
| y | Set dio |
| n | Normal play |
| p | Pause and play |
| t | stop record |
| k | Seek to start |
| e | Seek to end |
| d | Seek forward 5 second |
| a | Seek rewind 5 second |
| i | Setting recording information |
| Program number | Switch programs |