Montage LZ Linux Brief Sample

Sound Track User Guide

Document Number: LZ000444

Revision Number: 0.0

Revision Date: Jan. 4, 2024

Montage LZ Technologies Proprietary and Confidential

Revision History

| Revision Number | Revision Date | Changes |  |
| --- | --- | --- | --- |
|  |  | Page Number | Description |
| 0.0 | Jan. 4, 2024 | - | Initial Release |

Table of Contents

1	Overview	1

2	Sample Demo Information	2

2.1	Flowcharts	2

2.2	Function Modules	3

2.2.1	Switching Audio Channels	3

2.2.2	Switching Programs	3

2.3	Using API	3

2.3.1	mt_sys_init()	3

2.3.2	MTADP_Fe_Init()	3

2.3.3	MTADP_Fe_Connect_Dvbc()	3

2.3.4	MTADP_Fe_Connect_Dvbs()	3

2.3.5	MTADP_HDMI_Init ()	3

2.3.6	MTADP_Disp_Init()	4

2.3.7	MT_UNF_DISP_SetSmallWindow()	4

2.3.8	MTADP_VO_Init()	4

2.3.9	MTADP_Snd_Init()	4

2.3.10	MT_UNF_DMX_Init()	4

2.3.11	MT_UNF_DMX_AttachTSPort()	4

2.3.12	MTADP_Search_Init()	4

2.3.13	MTADP_Search_GetAllPmt()	5

2.3.14	MT_UNF_AVPLAY_Init()	5

2.3.15	MT_UNF_AVPLAY_Create()	5

2.3.16	MT_UNF_SND_CreateTrack()	5

2.3.17	MTADP_VO_CreatWin()	5

2.3.18	MT_UNF_VO_SetWindowEnable ()	5

2.3.19	MT_UNF_AVPLAY_Start ()	5

2.3.20	MT_UNF_AVPLAY_Stop ()	6

2.3.21	MT_UNF_VO_SetWindowEnable ()	6

2.3.22	MT_UNF_VO_DetachWindow ()	6

2.3.23	MT_UNF_VO_DestroyWindow ()	6

2.3.24	MT_UNF_SND_DestroyTrack ()	6

2.3.25	MT_UNF_AVPLAY_ChnClose ()	6

2.3.26	MT_UNF_AVPLAY_Destroy ()	6

2.3.27	MT_UNF_AVPLAY_DeInit ()	7

2.3.28	MTADP_Search_FreeAllPmt ()	7

2.3.29	MTADP_Search_DeInit ()	7

2.3.30	MT_UNF_DMX_DetachTSPort ()	7

2.3.31	MT_UNF_DMX_DeInit ()	7

2.3.32	MTADP_Snd_DeInit ()	7

2.3.33	MTADP_VO_DeInit ()	7

2.3.34	MTADP_Disp_DeInit ()	8

2.3.35	MTADP_HDMI_DeInit ()	8

2.3.36	MTADP_Fe_Deinit ()	8

2.3.37	mt_sys_deinit ()	8

3	Compilation	9

4	How to Use Sample Demo	10

4.1	Test Sample Environment Preparation	10

4.2	Test Single Board Environment Preparation	10

5	Supported Sample Commands	11

# 1	Overview

This sample primarily implements the functionality of switching audio channels. It provides guidance to clients when using related functions. Additionally, it offers simplified and structured invocation references during debugging.

# 2	Sample Demo Information

## 2.1	Flowcharts

## 2.2	Function Modules

### 2.2.1	Switching Audio Channels

Input command "f" to switch between different audio channels.

Invoke function MT_UNF_SND_SetTrackMode().

### 2.2.2	Switching Programs

Input program number to perform channel switching.

Invoke function MT_SoundTrackAVPlay_Start() for channel switching.

## 2.3	Using API

### 2.3.1	mt_sys_init()

[Information]

System device initialization; must be called, or various devices may not work correctly.

### 2.3.2	MTADP_Fe_Init()

[Information]

Initialize tuner device.

### 2.3.3	MTADP_Fe_Connect_Dvbc()

[Information]

About DVB-C frequency locking.

### 2.3.4	MTADP_Fe_Connect_Dvbs()

[Information]

About DVB-S frequency locking.

### 2.3.5	MTADP_HDMI_Init ()

[Information]

HDMI initialization.

### 2.3.6	MTADP_Disp_Init()

[Information]

Display device initialization.

### 2.3.7	MT_UNF_DISP_SetSmallWindow()

[Information]

Set window display size.

### 2.3.8	MTADP_VO_Init()

[Information]

VO device initialization.

### 2.3.9	MTADP_Snd_Init()

[Information]

Sound device initialization.

### 2.3.10	MT_UNF_DMX_Init()

[Information]

DMX initialization.

### 2.3.11	MT_UNF_DMX_AttachTSPort()

[Information]

Connect DMX to tuner device.

### 2.3.12	MTADP_Search_Init()

[Information]

Initialize search.

### 2.3.13	MTADP_Search_GetAllPmt()

[Information]

Retrieve PMT and PAT tables in TS.

### 2.3.14	MT_UNF_AVPLAY_Init()

[Information]

AV player initialization.

### 2.3.15	MT_UNF_AVPLAY_Create()

[Information]

Create AV player.

### 2.3.16	MT_UNF_SND_CreateTrack()

[Information]

Create audio track.

### 2.3.17	MTADP_VO_CreatWin()

[Information]

Create a window.

### 2.3.18	MT_UNF_VO_SetWindowEnable ()

[Information]

Open window display.

### 2.3.19	MT_UNF_AVPLAY_Start ()

[Information]

Start playing video.

### 2.3.20	MT_UNF_AVPLAY_Stop ()

[Information]

Stop playing video.

### 2.3.21	MT_UNF_VO_SetWindowEnable ()

[Information]

Close window display.

### 2.3.22	MT_UNF_VO_DetachWindow ()

[Information]

Detach window from player.

### 2.3.23	MT_UNF_VO_DestroyWindow ()

[Information]

Destroy window.

### 2.3.24	MT_UNF_SND_DestroyTrack ()

[Information]

Destroy audio track.

### 2.3.25	MT_UNF_AVPLAY_ChnClose ()

[Information]

Close audio and video channels.

### 2.3.26	MT_UNF_AVPLAY_Destroy ()

[Information]

Destroy AV player.

### 2.3.27	MT_UNF_AVPLAY_DeInit ()

[Information]

Deinitialize AV player.

### 2.3.28	MTADP_Search_FreeAllPmt ()

[Information]

Release PMT and PAT tables in TS.

### 2.3.29	MTADP_Search_DeInit ()

[Information]

Deinitialize search.

### 2.3.30	MT_UNF_DMX_DetachTSPort ()

[Information]

Detach DMX and tuner.

### 2.3.31	MT_UNF_DMX_DeInit ()

[Information]

Deinitialize DMX.

### 2.3.32	MTADP_Snd_DeInit ()

[Information]

Deinitialize audio device.

### 2.3.33	MTADP_VO_DeInit ()

[Information]

Deinitialize VO device.

### 2.3.34	MTADP_Disp_DeInit ()

[Information]

Deinitialize display.

### 2.3.35	MTADP_HDMI_DeInit ()

[Information]

Deinitialize HDMI.

### 2.3.36	MTADP_Fe_Deinit ()

[Information]

Deinitialize tuner.

### 2.3.37	mt_sys_deinit ()

[Information]

Deinitialize system.

# 3	Compilation

Execute 'make' for compilation.

# 4	How to Use Sample Demo

## 4.1	Test Sample Environment Preparation

Copy 'sample_switch_track' to a USB drive: linux\out\general\breif_sample\switch_track

Copy library files to the USB drive: linux\pub\shared_lib_striped\ (all files in the directory)

## 4.2	Test Single Board Environment Preparation

Connect the USB drive to the single board.

Execute the following commands:

mount /dev/sda1 /mnt/

export LD_LIBRARY_PATH=/mnt/shared_lib_striped/:/usr/local/lib

cd /mnt

./ sample_switch_track -f ./ttx.ts(file playback)

./ sample_switch_track -c 314  6875  64 -> frequency SymbolRate QAM(DVBC)

./ sample_switch_track -s 3840 27500 1 0 0 -> frequency SymbolRate 22k(0:off,1:on)(polarization mode)(DVBS type)

# 5	Supported Sample Commands

| Command | Description |
| --- | --- |
| q | Quit sample disp_zoom demo |
| h | Print support command |
| f | Switching sound tracks |
| Programme Number | Switch programs |